Kenwood TH-D74A/E

Accessible Guide

by Joseph Stephen VK5JKS

email: joestephen@internode.on.net

Updated February 28 2018.

Firmware version 1.07

Physical Description:

Left side, top to bottom

Top button, ptt.

Middle button, moni.

Bottom button, Power on/off, hold for a few seconds to turn on. Whilst on, a quick press of the power button will read band, mode, frequency and power, and activate the backlight.

2 dials on top right, inner and outer

Inside one is ENC control, changes channels, frequencies and menues.

Outer is volume.

Below the screen

Top left, mode (DTMF A).

Directly below is menu (DTMF B).

Centre, arrow keys plus enter in center.

Right top is A/B (DTMF C).

Directly below is F, or function button (DTMF D).

Below is a standard telephone style 12 key numeric keypad.

Right side, from the top down.

Speaker and mic.

Micro SD card slot.

Micro USB connector.

Power supply, charging port ((3.5 x 1.35 mm barrel, center positive).

Keypad functions:

1: VFO.

function + 1: memory to VFO (only works for analog channels).

2: Memory recall.

function + 2: Memory In.

3: Call channel;

function + 3, Call Channel In.

4: Msg;

function + 4, Enter new message input mode.

5: List Display APRS Station List.

function + 5: APRS on; KISS on; Off.

6: BCN. Press to transmit beacon when APRS is on.

function + 6: Transmit object.

7: Reverse.

function + 7: Select shift direction.

8: Tone/CTCSS/DCS.

function + 8: Set tone.

Hold function + 8 1s to perform a Tone scan.

9: PF1.

function + 9: Attenuator.

0: Mark Display Position Memory List.

function + 0: Display my position. Press and hold 0 to enter Mark Waypoint Registration Mode.

*: MHz Mode.

function + *: Enter Fine Tuning Mode. Press and hold to enter MHz Scan.

#: PF2 turns GPS on/off by default.

Function + [#] to enter Frequency step setup mode or Fine step frequency setup mode.

A: Mode.

Function + Mode: Digital Function Menu.

B: Menu.

function + B: Cycle TX Output Power.

C: A/B VFO.

function + C: Dual Band on/off.

function + Moni: Adjust squelch.

D: Function key, hold for 1 second to toggle keylock on and off.

To enter a frequency into VFO mode:

1. Press keypad 1 to go to VFO.

2. Press Enter (center of large square key.

3. Enter the frequency using the keypad. Use * for decimal if below 100 MHz (note only band B can tune to frequencies outside the ham bands).

4. If you enter less than all of the necessary digits, press Enter again. For example, if you entered 144 then press Enter, you'll be taken to 144.000 MHz. If you enter all the digits, you will be taken directly there without having to press Enter again. For example, if you enter 144000, you will not have to press Enter again.

To store an analog frequency in memory, follow the above steps to choose the frequency.

Then, if you need to, set the shift direction with Function + keypad 7.

Toggle the use of CTCSS/DCS/PL Tone code with keypad 8.

To set the code, press Function + Keypad 8, then use the knob to choose the actual code.

To store the frequency and settings to a memory:

1. press Function+keypad2 (Memory In),

2. choose a channel using the knob (all channels are spoken), then

3. press Enter to save the frequency and settings to that memory.

DStar functions

Before you can use DStar for the first time, there are two once off steps you need to perform.

1. You must register your callsign with the DStar network via the internet. This is usually done at your local DStar repeater’s website.

2. You must enter your callsign into the D74A radio using menu 610. When you choose that menu, you then use the keypad like a telephone keypad to choose the letters of your callsign. Press right arrow to advance the cursor if your radio is not set to advance it after you release a key.

You should also take the time to read some online DStar tutorials as this mode is sufficiently different to analog FM mode and we cannot cover it in depth in this Accessible Guide.

There are four pieces of data other than the actual frequencies needed to use DStar.

MYCall: Your own callsign, entered into the radio using menu 610, and registered on the DStar network via the Internet.

RPT1: The callsign and port of your local repeater through which your radio connects to the DStar network. For example, VK5RWN  B for 70 CM, or VK5RWN  C for 2 meters. Note the exact spacing, as these identifiers are exactly 8 characters long.

RPT2: The callsign and port through which your signal exits your local repeater. For example VK5RWN  G to send your signal out the Internet gateway (as well as via RF).

URCall: The callsign or special identifier specifying the final destination of your signal, usually set to CQCQCQ, or temporarily to a reflector ID and command character to link, unlink obtain repeater information, or to echo your own signal back to you.

Note that you can only use DStar on one band at a time. This means that if you have band B set to a DStar memory, you won't be able to choose DV/DR in VFO mode on band A. Similarly, if band A is set to a DStar memory or in VFO mode, you won't be able to choose DV/DR mode or a memory set to DV/DR mode on band B. Also, if a band is set to a memory channel with DStar parameters, if you go to VFO mode on that band, you won't be able to choose DV/DR mode. You first have to choose an analog channel before you can choose DV/DR in VFO mode.

To connect to a DStar repeater from the repeater list, do the following:

1. Press keypad 1 to go to VFO mode and choose a frequency in the ham band.

2. Cycle the mode to DR/DV mode using the Mode key.

2.1. If in DV mode, press Function+Menu to choose the digital Function Menu.

2.2. Choose option 5 to toggle to DR mode. You will automatically exit the menu when the setting is toggled.

3. Press and hold the down arrow 1 second to go to the From (FM) field menu. The choice you make from this menu will populate the RPT1 DStar data field. It will also usually put correct defaults in RPT2, and URCall fields.

4. You will be presented with a menu. You will be placed on the RList option, press Enter to choose it.

4.1. Use arrows to choose the world region and press Enter. (Note, the world regions from low beep are: Asia, Africa, Europe, North america, South america, Oceania.)

4.2. Use arrows to choose the country and press Enter. (Eg, if you chose Oceania, the countries from low beep are: Australia, New Zealand, Guam.)

4.3. Use arrows to choose the group and press Enter. (Note many countries only have one group at this level).

4.4. Finally use the arrows to choose the repeater and press Enter. (Note, at this level, all callsigns are spoken.)

If you want to use the GPS to locate a nearby DStar repeater, rather than using the RList option in step 3 above, you can down arrow to the Near (NR) option and press Enter.

If the GPS is on, (see menu 400), you will be placed in a list of the nearby repeaters. This list of callsigns talks just like the final step (4.4) of the long winded process of choosing world region, country, group and finally repeater described in 4.1 to 4.4 above.

Note, you may not need to set the To field. First check with Digital Function Menu option 4 to hear the four DStar fields. If they are correct, you can save to memory. If not, proceed to set the To field options also.

To check the fields, press Function + Mode, choose 4, then hit Enter. The four DStar fields will be spoken as:

U: cqcqcq

R2: VK5RWN g (for gateway)

R1: VK5RWN c (original repeater and port letter)

M: VK5JKS (your callsign)

Of course VK5RWN will be replaced by your chosen repeater, and VK5JKS by your callsign.

If you do need to set the To field,  press and hold up arrow 1 second to go to the TO field menu. The choice you make from this menu automatically populates either URCall, or both URCall and Rpt2 depending on which option you choose. The most useful options in this menu are R for reflector options and DI for direct input of the URCall field.

Once you've set up and verified the four fields, you can store this data like any analog repeater by following the three steps above this section, i.e. using Function + keypad 2 (Memory In), choose the channel with the knob, then press Enter to save the data.

More about the Digital Function Menu.

TO get to the Digital Function Menu, press Function + Mode.

(Note that the digital menu options are only spoken as a number so see below for more info)

DV Mode (fewer options):

1 Destination Select

4 CS Setting (reads UrCall, rpt1, rpt2 and MyCall.

5 DV/DR Select

6 Data Mode

7 TX History

9 Auto Reply

DR Mode:

1 Destination Select

2 Route Select

3 Repeater Detail

4 CS Setting (reads UrCall, rpt1, rpt2 and MyCall.

5 DV/DR Select

6 Data Mode

7 TX History

8 DR Scan

9 Auto Reply

If in DR (digital repeater mode):

Hold up arrow for 1s to go to the "To" field.

Hold down arrow 1s to go to the "From" field (spoken as fm).

From the From field:

low beep is RList: (full repeater list, see below),

NR: (nearby repeaters if you have GPS on) (see menu 400),

TxH: Tx history (choose from someone you last spoke to).

DI: Direct input,

From the To field:

low beep CQ: local cq,

G: gateway cq,

I: individual select,

R: Reflector menu (see below),

CH: Call history,

DI: Direct Input.

EMR: Emergency.

If you choose the repeater list (RList) from either the From or To fields, or the gateway cq option from the To field (g) you go straight to this menu:

w (world region)

Low beep is Asia,

Africa,

Europe,

North america,

South america,

Oceania (go up from low beep once to get here).

(then press Enter to go to country)

c (country)

Assuming you chose Oceania  above, the country options will be:

low beep Australia,

New Zealand,

Guam,

If you choose Australia, then

g (group)

low beep is Australia.

(Note, in many countries, there may only be one group so there may only be a low beep which you can then press Enter on to go to the repeater callsign list).

Once you get to the repeater callsign list, the callsigns speak as you arrow up and down.

Reflector Menu:

If you choose R from the To field (and press enter) to access the Reflector menu, the options are as follows:

low beep is Use reflector (assumes you are already linked),

link to reflector,

unlink reflector,

echo test,

reflector info,

If you choose Link to reflector (press enter), you can arrow between Ref, XRef and Dcs. if you choose one of these, you can then choose the number with up and down arrows, then press enter, then choose the module letter.

If you would like to hear the callsign of a calling DStar station, set menu 644 to all.

Unlike FM, it is often hard to know if you are making a DStar repeater because when you release the PTT button, there is no sound to indicate the repeater tail. TO enable a beep when you transmit and the repeater receives and acknowledges your signal, enable menu 645.

Channel List

To choose a channel using the keypad after you have programmed the channel memories:

1.	Ensure you are in Memory mode by pressing numpad 2.

2.	Press Enter to go to channel list.

3.	Enter a channel number using the keypad such as 001 (three digits).

4.	 Press Enter again and you'll leave the channel list and the radio

will be tuned to that channel.

If at step 3 above, you don't want to enter three digits, and only want to enter 1 or 2, the first time you press Enter after typing the digits, you'll move to that channel in the channel list but you will need to press Enter a 2nd time to exit the channel list to make the radio actually tune to that channel.

When in the Channel list, and you press Menu, you will be taken to the channel menu rather than the main menu.

From this menu you can edit a memory, shift a memory's content to VFO, delete a channel, etc.

If you choose Memory Edit, the resultant menu doesn't speak, but the first option after the low beep is Edit Channel name. Use the keypad to edit the name of the channel and press Enter.

When editing text, the band a/b key is used to delete the current character.

MENU CONFIGURATION

Please note, when you press the menu key, you can type in the three digit menu option on the numeric keypad to go straight to that menu.

You can also use the arrows to select a menu number and press enter to go to the next menu level.

Please also note that by default, when you enable voice guidance by holding # and turning on the power, that voice guidance is set to auto1. If you edit the names of channels, and wish to hear the name rather than the frequency for named channels, change menu 916 to auto2.

No.; Display; Description; Setting Values

TX/RX - RX

100: Programmable VFO; Programmable VFO setting (Band A only); Varies with the selected frequency band

101: Beat Shift; Beat shift; Type 1 - Type 8

102: Detect Out Select; Detect output select; Off (AF)/ IF/ Detect

103: FM Narrow; FM narrow; Off/ On

104: MW/ SW Antenna; MW/ SW Antenna; ANT connector / Bar Antenna

105: WX Alert; Weather alert; Off/ On (TH-D74A only)

TX/RX - TX

110: TX Inhibit; TX inhibit; Off/ On

111: Time-out Timer; Time-out timer; 0.5/ 1.0/ 1.5/ 2.0/ 2.5/ 3.0/ 3.5/ 4.0/ 4.5/ 5.0/ 10.0 [min]

112: Mic. Sensitivity; Microphone sensitivity; High/ Medium/ Low

TX/RX - RX Filter

120: SSB High Cut; SSB high cut frequency; 2.2/ 2.4/ 2.6/ 2.8/ 3.0 [kHz]

121: CW Width; CW bandwidth; 0.3/ 0.5/ 1.0/ 1.5/ 2.0 [kHz]

122: AM High Cut; AM high cut frequency; 3.0/ 4.5/ 6.0/ 7.5 [kHz]

TX/RX - Scan

130: Resume; Resume method; Time/ Carrier/ Seek

131: Resume (Digital); Resume method (Digital); Time/ Carrier/ Seek

132: Time Restart; Time operate restart time; 1 - 5 - 10 [sec]

133: Carrier Restart; Carrier operate restart time; 1 - 2 - 10 [sec]

134: Priority Scan; Priority scan; Off/ On

135: Scan Auto Backlight; Scan auto backlight; Off/ On

136: Auto Weather Scan; Auto Weather Channel Scan; Off/ On (TH-D74A only)

TX/RX - Repeater

140: Offset Frequency; Offset frequency; Varies with the selected frequency band

141: Auto Offset; Auto repeater offset; Off/ On

142: CALL Key; CALL key function; CALL (TH-D74A)/ 1750Hz (TH-D74E)

143: 1750Hz TX Hold; 1750 Hz TX hold; Off/ On

TX/RX - VOX

150: VOX; VOX on/ off; Off/ On

151: Gain; VOX gain level; 0 - 4 - 9

152: Delay; VOX delay time; 250/ 500/ 750/ 1000/ 1500/ 2000/ 3000 [ms]

153: TX on Busy; VOX on busy; Off/ On

TX/RX - DTMF

160: Encode Speed; Encode speed; 50/ 100/ 150 [ms]

161: Pause Time; Pause time; 100/ 250/ 500/ 750/ 1000/ 1500/ 2000 [ms]

162: TX Hold; TX hold; Off/ On

163: DTMF Memory; DTMF memory; Up to 10 channels for DTMF memory channel

Up to 16 characters for DTMF memory name

Up to 16 digits for DTMF memory code

164: EchoLink Memory; EchoLink memory; Up to 10 channels for EchoLink memory channel

Up to 8 characters for EchoLink memory name

Up to 8 digits for one channel code

TX/RX - CW

170: Pitch Frequency; Pitch frequency; 400 - 800 - 1000 [Hz]

171: Reverse; Reverse; Normal/ Reverse

TX/RX - Others

180: QSO Log; QSO log; Off/ On

181: LED Control; LED control; RX: Check, FM Radio: Uncheck

Memory - Memory Channel

200: View List; Memory channel list; -

201: Group Name; Memory group name input; Up to 16 characters

202: Recall Method; Memory channel recall method; All Bands/ Current Band

203: Group Link; Memory group link registration; register up to 30 memory group links

204: CALL Ch List; CALL channel list; -

Memory - Repeater List

210: View List; Repeater list; -

Memory - Callsign List

220: View List; Callsign list; -

Audio File - Recording File

300: View List; Recording file list ; -

301: Recording; Recording; Off/ On

302: Recording Band; Recording band; A Band/ B Band

Audio File - Voice Message

310: View List; Voice message list; -

311: TX Monitor: TX monitor: Off / On

312: Digital Auto Reply; Digital auto reply; Off/ Voice Message 1 - Voice Message 4

GPS - Basic Settings

400: Built-in GPS: Built-in GPS; Off/ On

401: My Position; My position; GPS/ My Position 1 - 5

402: Position Ambiguity; Position ambiguity mode; Off/ 1-Digit - 4-Digit

403: Operating Mode; Built-in GPS operating mode; Normal/ GPS Receiver

404: Battery Saver; Battery saver time; Off/ 1min/ 2min/ 4min/ 8min/ Auto

405: PC Output; GPS data output to PC; Off/ On

406: Sentence; Sentence; $GPGGA/ $GPGLL/ $GPGSA / $GPGSV/ $GPRMC/ $GPVTG

GPS - Track Log

410: Track Log; Track log recording; Off/ On

411: Clear Track Log; Clear track log; -

412: Record Method; Record method; Time/ Distance/ Beacon

413: Interval; Interval time; 2 - 10 - 1800 [sec]

414: Distance; Distance; 0.01 - 9.99 [mile/ km/ nm]

APRS - Basic Settings

500: My Callsign; Callsign entry; Up to 9 characters

501: Icon; Icon; Person/ Bicycle/ Motorcycle, etc. (total 68 icons)

502: Position Comment; Position comment; Off Duty/ Enroute/ In Service/ Returning/ Committed/ Special/ PRIORITY/ CUSTOM0 ~ CUSTOM6/EMERGENCY!

503: Status Text; Status text; Status text: 1 - 5 TX Rate: Off/ 1/1 - 1/4 - 1/8

Up to 42 characters

504: Packet Path; Packet path type; Type: New-N/ Relay/ Region/

Others1-Others3, WIDE1-1: Off/On, RELAY: Off/On, ABBR: Up to 5 characters, Total Hops: 0 - 1 - 7, Path: Up to 79 characters

505: Data Speed; Data communications speed; 1200bps/ 9600bps

506: Data Band; Internal data band type; A Band/ B Band

507: DCD Sense; DCD sense type; Busy/ Detect Data/ Off (Ignore)

508: TX Delay; TX delay time; 100/ 150/ 200/ 300/ 400/ 500/ 750/ 1000 [ms]

509: APRS Lock; APRS lock; Frequency/ PTT/ APRS Key: All unchecked

APRS - Beacon TX Control

510: Method; Method; Manual/ PTT/ Auto/ SmartBeaconing

511: Initial Interval; Initial Interval timer; 0.2/ 0.5/ 1/ 2/ 3/ 5/ 10/ 20/ 30/ 60 [min]

512: Decay Algorithm; Decay Algorithm; Off/ On

513: Prop. Pathing; Prop. Pathing; Off/ On

514: Speed; Speed; Off/ On

515: Altitude; Altitude ; Off/ On

516: Object; Object name; Name: up to 9 characters, Type: Live Object/ Killed Object/ Live Item/ Killed Item, Method: Off/ Temp./ Auto(15 min)/ Auto(30 min)/ Auto(60 min), N(S): Latitude, E(W): Longitude, Icon (Total 68 kinds): Eyeball/ Portable (Tent)/ HAM store, etc., Comment: up to 42 characters

APRS - QSY Information

520: QSY Info. in Status; QSY information in status; Off/ On

521: Tone/Narrow; Tone/ Narrow; Off/ On

522: Shift/Offset; Shift/ Offset; Off/ On

523: QSY Limit Distance; QSY limit distance; Off/ 10/ 20 … 2490/ 2500 [mile/ km/ nm]

APRS - SmartBeaconing

530: Low/High Speed; Low speed/ High speed setting; Low Speed: 2 - 5 -

30 [mile/h/ km/h/ knots] High Speed: 2 - 70 - 90 [mile/h/ km/h/ knots]

531: Slow Rate; Low speed transmission interval time; 1- 30 - 100 [min]

532: Fast Rate; High speed transmission interval time; 10 - 120 - 180 [sec]

533: Turn Angle; Driving direction change, minimum value setting; 5 deg - 28 deg - 90 deg

534: Turn Slope; Driving direction change, additional value setting; 1 (10deg/speed) - (26 10deg/speed) - 255 (10deg/speed)

535: Turn Time; Minimum time delay between each beacon transmission; 5 - 60 - 180 [sec]

APRS - Waypoint

540: Format; Way point format; NMEA/ MAGELLAN/ KENWOOD

541: Length; Way point name length; 6-Char/ 7-Char/ 8-Char/ 9-Char

542: Output; Way point output type; All/ Local/ Filtered

APRS - Packet Filter

550: Position Limit; Position limit; Off/ 10/ 20 … 2490/ 2500 [mile/ km/ nm]

551: Filter Type; Filter type; Weather/ Digipeater/ Mobile/ Object/ NAVITRA/ 1-WAY/ Others

APRS - Message

560: User Phrase; User phrases; Up to 32 characters x 20 phrases

561: Auto Reply; Auto message reply; Off/ On

562: Reply To; Reply to; Up to 9 characters

563: Reply Delay Time; Reply delay time; 0/ 10/ 20/ 30/ 60 [sec]

564: Reply Message Text; Reply message text input; Up to 50 characters

APRS - Notification

570: RX Beep; RX beep; Off/ Message Only/ Mine/ All New/ All

571: TX Beep; TX beep; Off/ On

572: Special Call; Special call; Up to 9 characters

573: Display Area; Display area; Entire Always/ Entire Display/ One Line

574: Interrupt Time; Interrupt time; 3/ 5/ 10/ 20/ 30/ 60/ infinite [sec]

575: APRS Voice; APRS voice; Off/ On

APRS - Others

580: PC Output; PC output type; Off/ Raw Packets/ Waypoints

581: Network; Network type; APRS[APK004]/ Altnet

582: Voice Alert; Voice alert type; Off/ On/ RX Only

583: VA Frequency; VA frequency type; 67.0 - 100.0 - 254.1 Hz

584: Message Group Code; Message group code; Up to 9 characters x 6 codes (ALL,QST,CQ,KWD)

585: Bulletin Group Code; Bulletin group code; Up to 5 characters x 6 codes

Digital - RX History

600: View History; View History; -

Digital - TX/RX

610: My Callsign; Callsign entry; Up to 8 characters + up to 4 characters

611: TX Message; TX message; Off/ 1/ 2/ 3/ 4/ 5

612: Direct Repl-y; Direct reply; Off/ On

613: Auto Reply Timing; Auto reply timing; Immediate/ 5/ 10/ 20/ 30/ 60 [sec]

614: Data TX End Timing; Data TX end timing; Off/ 0.5/ 1/ 1.5/ 2 [sec]

615: EMR Volume Level; EMR Volume level; 1 - 25 - 50

616: RX AFC; RX AFC; Off/ On

617: FM Auto Det. on DV; FM auto detector on DV; Off/ On

618: Data Frame Output; Data Frame Output; All/ Related to DSQ/ DATA Mode

619: Break Call; Break Call; Off/ On

Digital - Digital Squelch

620 Select Type Select Type Off/Code Squelch/ Callsign Squelch

621 Digital Code Digital Code 00 - 99

Digital - GPS Data TX

630 GPS Info. in Frame GPS Information in frame Off/ On

631 Sentence Sentence $GPGGA/ $GPGLL/ $GPGSA/ $GPGSV/ $GPRMC/

$GPVTG

632 Auto TX Auto TX Off/ 0.2/ 0.5/ 1/ 2/ 3/ 5/ 10/ 20/ 30/ 60 [min]

Digital - RX Notification

640 Display Method Display method Off/ All/ Related to DQS/ My Station Only

641 Single Display Size Single display size Half Display/ Entire Display

642 Dual Display Size Dual display size Half Display/ Entire Display

643 Display Hold Time Display hold time 0 / 3/ 5/ 10/ 20/ 30 / 60/ Infinite [sec]

644 Callsign Announce Callsign announce Off/ Kerchunk/ Except Kerchunk/ My Station Only/ All

645 Standby Beep Standby beep Off/ On

FM Broadcasting - Basic Settings

700 FM Radio Mode FM radio mode Off/ On

701 Auto Mute RET. Time Auto mute return time 1 - 3 - 10 [sec]

FM Broadcasting - Memory

710 FM Radio List FM radio list -

SD Card - Export

800 Config Data Config data -

801 Config Data + V.Msg Config data + V.msg -

802 Repeater List Only Repeater list -

803 Callsign List Only Callsign list -

SD Card - Import

810 Config Data Config data -

811 Config Data + V.Msg Config data + V.msg -

812 Repeater List Only Repeater list -

813 Callsign List Only Callsign list -

SD Card - Unmount

820 Execute Unmount execute -

SD Card - Format

830 Execute Format execute -

SD Card - Memory Size

840 View Free capacity  -

Configuration - Display

900 Backlight Control Backlight control Auto/ Auto (DC-IN)/ Manual/ On

901 Backlight Timer Backlight timer 3 - 10 - 60 [sec]

902 LCD Brightness LCD brightness High/ Medium/ Low

903 Power-on Message Power-on message input Up to 16 characters

904 Single Band Display Single band display type Off/ GPS(Altitude) / GPS(GS)/ Date

905 Meter Type Meter type Type 1/ Type 2/ Type 3

906 Background Color Background color select Black/ White

Configuration - Audio

910 Balance Audio balance

A:100/ B:0, A:100/ B:25, A:100/ B:50, A:100/ B:75, A:100/ B:100, A:75/ B:100, A:50/ B:100, A:25/ B:100, A:0/B:100,

Operation Band Only

911 TX/RX EQ TX/RX EQ RX EQ/ TX EQ(FM, NFM)/ TX EQ(DV)

912 TX EQ Level TX EQ Level -9 - ±0 - +3 [dB]

913 RX EQ Level RX EQ Level -9 - ±0 - +9 [dB]

914 Beep Beep Off/ On

915 Beep Volume Beep Volume Level 1 - Level 5 - Level 7

916 Voice Guidance Voice Guidance Off, Manual, Auto1, Auto2

917 Voice Guidance Vol. Voice Guidance Vol. Level 1 - Level 5 - Level 7

918 USB Audio Out. Lvl. USB Audio Output level Level 1 - Level 5 - Level 7

Configuration - Battery

920 Battery Saver Battery Saver Off/ 0.2/ 0.4/ 0.6/ 0.8/ 1.0/ 2.0/ 3.0/ 4.0/ 5.0 [sec]

921 APO: Auto Power Off APO: Auto Power Off Off/ 15/ 30/ 60 [min]

922 Battery Level Battery Level -

Configuration - Bluetooth

930 Bluetooth Bluetooth Off / On

931 Connect Connect -

932 Device Search Device Search -

933 Disconnect Disconnect -

934 Pairing Mode Pairing Mode -

935 Device Information Device Information Up to 19 characters

936 Auto Connect Auto Connect Off / On

Configuration - Auxiliary

940 PF1 Key PF1 Key Recording - Voice Message 1-4 - Voice Guidance - Battery

Level - VOX -  Group Name - Balance (PF1) - GPS (PF2) - Track LOG - SQL - SHIFT - STEP - LOW - Key

Lock - Lockout - M>V - T. SEL - NEW - Voice Alert - LCD

Brightness - DTMF CH0 - EchoLink CH0 - 1750Hz Tone

- M. IN

941 PF2 Key PF2 Key

942 PF1 (Mic) PF1 (Mic) Recording - Voice Message 1-4 - Voice Guidance - Battery

Level - VOX -  Group Name - Balance - GPS - Track LOG -

SQL - SHIFT - STEP - LOW - Key Lock - Lockout - M>V -

T. SEL - NEW - Voice Alert - LCD Brightness - DTMF CH0

- EchoLink CH0 - 1750Hz Tone - Screen Capture - MODE

- MENU - A/B (PF1 Mic) - VFO (PF2 Mic) - MR (PF3 Mic) - CALL- MSG - LIST - BCON - REV - TONE - MHz -

MARK - DUAL - APRS - OBJ - ATT - FINE - POS - BAND

- MONI - UP - DOWN

943 PF2 (Mic) PF2 (Mic)

944 PF3 (Mic) PF3 (Mic)

945 Cursor Shift Cursor shift Off/ 1.0/ 1.5/ 2.0 [sec]

946 Secret Access Code Secret access code input 000 - 999 (TH-D74A only)

Configuration - Date & Time

950 Setting Date and time setting -

Configuration - Lock

960 Keys Lock Type Keys lock type Key Lock/ Frequency Lock

961 DTMF Keys Lock DTMF keys lock Off/ On

962 Mic Keys Lock Microphone keys lock Off/ On

963 Volume Lock Volume lock Off/ On

Configuration - Units

970 Speed, Distance Speed/ Distance mi/h, mile (TH-D74A)/ km/h, km (TH-D74E)/ knots, nm

971 Altitude, Rain Altitude/ Rain feet, inch (TH-D74A)/ m, mm  (TH-D74E)

972 Temperature Temperature °F (TH-D74A)/ °C (TH-D74E)

973 Latitude, Longitude Latitude/ Longitude dd°mm.mm'/ dd°mm'ss.s"

974 Grid Square Format Grid square format Maidenhead Grid/ SAR Grid (CONV)/ SAR Grid (CELL)

Configuration - Interface

980 USB Function USB Function COM+AF/IF Output/ Mass Storage

981 PC Output(GPS) PC Output(GPS) USB/ Bluetooth

982 PC Output(APRS) PC Output(APRS) USB/ Bluetooth

983 KISS PC Input/ Output(KISS) USB/ Bluetooth

984: DV/DR; PC Input/ Output(DV/DR) USB/ Bluetooth

Configuration - System

990: Language; Language; English/ Japanese

991: Version; Firmware version; -

999: Reset; Reset; VFO Reset/ Partial Reset/ Full Reset